For pjsip

Pjsip Registration:

[22252318]

type=registration

transport=0.0.0.0-udp

outbound_auth= 22252318

retry_interval=60

fatal_retry_interval=30

forbidden_retry_interval=30

max_retries=10000

expiration=3600

auth_rejection_permanent=no

line=yes

endpoint= 22252318

server_uri=sip:trunk01.cypbx.eu:5060

client_uri=sip:22252318@trunk01.cypbx.eu:5060

pjsip identification

[22252318]

type=identify

endpoint=22252318

match= sip:trunk01.cypbx.eu

pjsip authentication

[22252318]

type=auth

auth_type=userpass

password=qPBXZfXu08jNtu27U1jY

username=22252318

pjsip AOR

[22252318]

type=aor

qualify_frequency=60

contact=sip:22252318@trunk01.cypbx.eu:5060

pjsip.conf

[22252318]

type=endpoint

transport=0.0.0.0-udp ;---- or your transport IP;

context=from-trunk

disallow=all

allow=alaw,

aors=22252318

send_connected_line=false

language=en_GB

outbound_auth=22252318

from_domain=trunk01.cypbx.eu

user_eq_phone=no

t38_udptl=no

t38_udptl_ec=none

fax_detect=no

trust_id_inbound=no

t38_udptl_nat=yes

direct_media=no

rtp_symmetric=yes

dtmf_mode=rfc4733

NOTE: some changes in Nat setting might be needed based on client config

SIP Registration Details

[22252318]

defaultuser=22252318

type=peer

secret=qPBXZfXu08jNtu27U1jY

insecure=port,invite

host=trunk01.cypbx.eu

context=from-trunk

[Trunk01_22252318]

defaultuser=22252318

type=peer

secret=qPBXZfXu08jNtu27U1jY

qualify=yes

host=trunk01.cypbx.eu

context=from-trunk (OR what the client needs)

register=22252318:qPBXZfXu08jNtu27U1jY@trunk01.cypbx.eu/22252318